# HSRP Configuration Guide

# Overview

HSRP (Hot Standby Router Protocol) provides gateway redundancy by allowing multiple routers to share a virtual IP address.

# Topology Example

R1: 192.168.1.1 (Active preferred)
R2: 192.168.1.2 (Standby)
Virtual IP: 192.168.1.254

# R1 Configuration

interface g0/0
 ip address 192.168.1.1 255.255.255.0
 standby 1 ip 192.168.1.254
 standby 1 priority 110
 standby 1 preempt

# R2 Configuration

interface g0/0
 ip address 192.168.1.2 255.255.255.0
 standby 1 ip 192.168.1.254
 standby 1 priority 100
 standby 1 preempt

# Verification

show standby

# Key Points

Virtual IP is used as default gateway. Preempt allows higher priority router to regain active role. Failover is automatic.